ANNEXURE A: TECHNICAL SPECIFICATION

BID NUMBER RT63-2022

SUPPLY AND DELIVERY COVID-19 TEST KITS TO THE STATE FOR THE PERIOD OF 24 MONTHS

NATIONAL TREASURY

TRANSVERSAL CONTRACTING

Table of Contents

TECHNICAL SPECIFICATION	3

1.	MANDATORY REQUIREMENTS	3

2.	TECHNICAL DETAILS	3

# TECHNICAL SPECIFICATION

# MANDATORY REQUIREMENTS

  - Rapid antigen test –SAHPRA test kit approval for use in South Africa

  - Suppliers must supply a batch from each new lot number for verification at the national reference laboratory

  - All required consumables should be supplied within or with the kit

  - Sensitivity should be documented at >85% and specificity >98%

  - The item awarded must be able to be adaptive of updated technical specification as published by SAHPRA to cater for the new discovered Covid-19 variants

  - The state reserve the right to suspend procurement of the awarded testing kits which no longer serve effectively to detect the Covid-19 variant during the contract period.

# TECHNICAL DETAILS

  - Test performance 40%

- Published sensitivity 85-89% 4 points

- Published sensitivity 90%-94% 8 points

- Published sensitivity 95% + 12 points

- Published specificity 98% 8 points

- Published specificity >98% 8 points

  - Ease of use 20%

- Preanalytical processing required (including cutting swabs, addition of buffer or other processing) AND/OR open system requiring separate disposal of the swab 10%

- Preanalytical processing required (including cutting swabs, addition of buffer or other processing) BUT Closed system 15%

- Minimal preanalytical processing required and closed system (addition of buffer by pipette or dropper) 20%

  - Requirements of analyser or reader 10%

- Analyser required and supplied 5%

- No analyser required (i.e. lateral flow assay) 10%

  - Training 10%

- No training provided 0%

- Access to online training material 5%

- Onsite technical training 10%

  - Specimen type 20%

- Can be utilised on nasopharyngeal swabs only 10%

- Can be utilised on nasopharyngeal swabs and oropharyngeal swabs 15%

- Can be utilised on sputum (for use on an early morning sample prior to eating/drinking/brushing teeth), nasopharyngeal swabs and oropharyngeal swabs 20%