| 1 | TECHNICAL SPECIFICATION DOORS & GATES: | COMPLY: YES/NO |
| --- | --- | --- |
| 1.1 | TECHNICAL SPECIFICATIONS TO MANUFACTURE, SUPPLY AND DELIVER, GALVANIZED STEEL CELL DOORS, GRILLE GATES AND FRAME, SIZE OF UNIT : WIDTH 1018 MM HEIGHT 2020 MM AND / OR WIDTH 1208 MM HEIGHT 2020 MM | STATE: _________________ |
| 1.1.1 | All items will be manufactured in accordance to drawings: POL 2012/D1; POL 2012/D1a; POL 2012/D2; POL 2012/D3; POL 2012/D4 and POL 2012/D5. The cell doors and gate lock boxes have a key hole only on the exercise yard side. Cell side to be blocked off. | STATE: _________________ |
| 1.1.2 | All manufactured steel cell door, gate and frames units will be HOT DIPPED Galvanized and shall comply with the requirements of all applicable SANS 121 (SABS ISO 1461) set standards. | STATE: _________________ |
| 1.1.3 | The galvanized steel cell door, gate and frames may be requested as follows: Right hand cell door and a left hand cell gate unit(s) Left hand cell door and right hand cell gate unit(s) | STATE: _________________ |
| 1.1.4 | Delivery to be made within twenty one (21) working days after official orders has been received by the successful bidder. | STATE:_________________ |
| 1.2 | INFO ON DRAWING POL 2012 / D1 RECEIVED |  |
| 1.2.1 | 6. 4 mm x 16 mm monobolts both sides of door at 120 mm maximum centres. | STATE:_________________ |
| 1.2.2 | 16 mm diameter Bright mild steel rods EN 8. 200 mm x 25 mm x 2 mm mild steel lugs welded on angle iron 50 mm x 50mm 6 mm. | STATE:_________________ |
| 1.2.3 | Opening for meals grille max 300mm | STATE:_________________ |
| 2.2.4 | Bow handle out of 40 mm x 5 mm mild steel weld and fixed with 6, 4 x 22 mm monobolts to frame close cut out on the outside of door only. | STATE:_________________ |
| 1.2.5 | Slotted in angle for striker; Cell-type lock box | STATE:_________________ |
| 1.3 | INFO ON DRAWING POL 2012 / D1a RECEIVED | STATE:_________________ |
| 1.3.1 | 6.4 mm x 16 mm monobolts both sides of door at 120 mm maximum centres. | STATE: _________________ |
| 1.3.2 | Diameter 16 mm Bright mild steel rods EN 8. | STATE:_________________ |
| 1.3.3 | 200 mm x 25 mm x 2 mm mild steel lugs welded to steel frame accordingly, max spacing not less than 300mm between lugs. | STATE:_________________ |
| 1.3.4 | Bow handle out of 40 mm x 5 mm mild steel weld and fixed with 6, 4 x 22 mm monobolts to frame close cut out on the outside of door only. | STATE: _________________ |
| 1.3.5 | Expanded metal Screen on Solid door (flatex 345 VEM 6320K) Slotted in for inspections only where requested. | STATE:_________________ |

| 1.4 | INFO ON DRAWING POL 2012 / D 2 RECEIVED | STATE:_________________ |
| --- | --- | --- |
|  | FRAMES: |  |
| 1.4.1 | Frame manufactured from steel angles and 2 mm thick steel sheet shaped into sizes as per detail drawings. | STATE:_________________ |
|  | HINGES: |  |
| 1.4.2 | 3 Hinges per leaf with middle hinge upside down as per Hinge detail. | STATE:_________________ |
|  | DOORS: |  |
| 1.4.3 | Doors & Grille manufactured from steel tubing, bars and steel sheet metal shaped into sizes as per detail. | STATE:_________________ |
|  | GRILLE GATES: |  |
| 1.4.4 | Grille manufactured from steel tubing, bars and shaped into sizes as per detail. | STATE:_________________ |
|  | LOCKS & LOCK BOXES: |  |
| 1.4.5 | Lockboxes must be supplied and provided by the bidder and welded into the Door & Gates. The Bidder must make provision in his/her price for collection of the Lock from Pretoria. Supply of locks to be coordinate and acquired by SAPS Project and Building Management Pretoria against the contract Budget. | STATE:_________________ |
|  | HANDLES: |  |
| 1.4.6 | Handles as indicated on drawings to be supplied only to the outside of the solid door leaf. | STATE:_________________ |

| 1.5 | INFO ON DRAWING POL 2012 / D 3 RECEIVED | STATE:_________________ |
| --- | --- | --- |
|  | PLAN SECTION B – B |  |
| 1.5.1 | 40 mm x 8 mm flat steel welded to angle iron to attach hinge as shown. Cut aperture into sheet metal and weld directly onto tubing. | STATE:_________________ |
| 1.5.2 | 200 mm x 25 mm x 2 mm mild steel lugs welded onto frame to be built into every 4 brick courses. | STATE:_________________ |
| 1.5.3 | 40 mm x 8 mm flat steel welded directly on square tubing frame. Cut aperture into sheet metal and weld directly on to tubing. Close cut out with welding joint. | STATE:_________________ |
| 1.5.4 | 50mm x 50 mm x 4 mm hollow steel tubing. | STATE:_________________ |
| 1.5.5 | 2 mm sheet metal door frame. | STATE:_________________ |
| 1.5.6 | 16mm dia. Bright mild steel rods EN 8. | STATE:_________________ |
| 1.5.7 | 2 mm sheet metal riveted to frame with monobolts on both sides. | STATE:_________________ |
| 1.5.8 | 50 mm x 50 mm 6 mm angle iron. | STATE:_________________ |
| 1.5.9 | 17 mm x 60 mm Aperture in frame and 50 x50 x6 mm angle iron at centre of lock box position for lock bolt with a 65 x 20 x 25 mm long mortar box. | STATE: _________________ |
| 1.5.10 | Shaft diameter 6 mm monobolts to hinge side of doors at maximum centres of 120 mm | STATE:_________________ |

| 1.6 | INFO ON DRAWING POL 2012 / D 4 RECEIVED | STATE:_________________ |
| --- | --- | --- |
| 1.6.1 | Fit top & bottom hinges to cell door or grille gate to allow for sufficient play for at least 4 mm play either up or down. | STATE:_________________ |
| 1.6.2 | Adjust setting bolts to the specified top gap between leaf and frame. There after weld middle hinge (upside down as shown) into place allowing a space of 4 mm between the two parts for further future adjustments. | STATE:_________________ |
| 1.6.3 | 50 x 50 x 6 mm angle iron. | STATE:_________________ |
| 1.6.4 | 2 mm sheet metal door frame. | STATE:_________________ |
| 1.6.5 | 50 x 50 x 4 mm hollow section profile. 2 mm sheet metal fixed onto frame with 6 x 16 mm monobolts at max. 120 centres on both sides. | STATE:_________________ |
| 1.6.6 | Bright mild steel EN 8. | STATE:_________________ |
| 1.6.7 | Lock box to be welded in during construction of door or gate. | STATE:_________________ |
| 1.6.8 | Bow handle out of 40 mm x 5 mm mild steel weld and fixed with 6, 4 x 22 mm monobolts to frame close cut out on the outside of door only. | STATE:_________________ |

| 1.7 |  | INFO ON DRAWING POL 2012 / D5 RECEIVED |  | STATE:_________________ |
| --- | --- | --- | --- | --- |
| 1.7.1 |  | All lock boxes to be made and installed by the manufacturer, SAPS Project and Building Management will only be responsible for the supply of the cell lock on request through the correct channels. All boxes to be welded into the Door or Gate frames. |  | STATE:_________________ |
| 1.7.2 |  | The Cell door & gate lock box to have a keyhole only on the exercise yard side. Cell side to be blocked off. |  | STATE:_________________ |
|  |  | LOCK BOX – FRONT ELAVATION |  |  |
| 1.7.3 |  | 25 mm x 5 mm x 40 mm mild steel lip welded to inside of lockbox as shown with 10 mm x 1,5 mm threaded hole to match position of lock. |  | STATE: _________________ |
| 1.7.4 |  | 40 mm x 4 mm thick mild steel back enclosure. |  | STATE: _________________ |
| 1.7.5 |  | 16 mm dia. Bright mild steel EN 8 round bar welded to inside. |  | STATE:_________________ |
| 1.7.6 |  | 40 mm x 12 mm mild steel flat bar |  | STATE:_________________ |
|  |  | LOCK BOX – TOP ELAVATION |  |  |
| 1.7.7 |  | 5 mm thick mild steel for sides16 mm dia. |  | STATE:_________________ |
| 1.7.8 |  | Bright mild steel EN 8 round bar welded to inside 40 mm x 40 mm thick mild steel back enclosure. |  | STATE:_________________ |
| 1.7.9 |  | 25 mm x 25 mm x 40 mm mild steel lip welded to inside of lock box top elevation as shown. |  | STATE:_________________ |
| 1.7.10 |  | 10 x 1. 5 mm thread countersank flat head bolts |  | STATE:_________________ |
| 1.7.11 |  | Picture, brochure and/or photo of the product will be submitted with the Bid / Term Tender submission. |  | STATE:_________________ |
| 1.8 |  | CELL STEEL RIGHT-HAND DOOR, LEFT-HAND GATE AND FRAME UNIT(S) HEIGHT 2020MM WIDTH 1018 MM; FOR CELL PASSAGES, DUCTING’S, YARDS AND/OR CELLS areas to be manufactured, hot dipped galvanized, supplied, delivered and offloaded according to set drawings POL 2022 / D1, POL 2022 / D2, POL 2022 / D3, POL 2022 / D4 and POL 2022 / D5 DETAILOF CELL DOOR GATE AND FRAME as per set drawings and specifications. WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED ICN Code: 9825T05073084 |  | STATE: _________________ |
| 1.9 |  | CELL STEEL LEFT-HAND DOOR, RIGHT-HAND GATE AND FRAME UNIT(S), HEIGHT 2020MM WIDTH 1018 MM; FOR CELL PASSAGES, DUCTING’S, YARDS AND/OR CELLS areas to be manufactured, hot dipped galvanized, supplied, delivered and offloaded according to set drawings POL 2022 / D1, POL 2022 / D2, POL 2022 / D3, POL 2022 / D4 and POL 2022 / D5 DETAILOF CELL DOOR GATE AND FRAME as per set drawings and specifications WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED ICN Code: 9825T05073085 |  | STATE:_________________ |
| 1.10 |  | CELL STEEL RIGHT-HAND DOOR, LEFT-HAND GATE AND FRAME UNIT(S), HEIGHT 2020MM, WIDTH 1208 MM; FOR CELL KITCHEN AND/OR DISABLED to be manufactured, hot dipped galvanized, supplied, delivered and offloaded according to set drawings POL 2022 / D1a, POL 2022 / D2, POL 2022 / D3, POL 2022 / D4 and POL 2022 / D5 DETAILOF CELL DOOR GATE AND FRAME as per set drawings and specifications WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED ICN Code: 5340T05073079 |  | STATE: _________________ |
| 1.11 |  | CELL STEEL LEFT -HAND DOOR, RIGHT-HAND GATE AND FRAME UNIT(S), HEIGHT 2020MM, WIDTH 1208 MM; FOR CELL KITCHEN AND/OR DISABLED areas to be manufactured, hot dipped galvanized, supplied, delivered and offloaded according to set drawings POL 2022 / D1a, POL 2022 / D2, POL 2022 / D3, POL 2022 / D4 and POL 2022 / D5 DETAILOF CELL DOOR GATE AND FRAME as per set drawings and specifications. WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED ICN Code: 5340T05073080 |  | STATE: _________________ |
| 2 |  | MANUFACTURED OF CELL DOOR HINGES TOP, MIDDLE AND BOTTOM UNITS |  |
| 2.1 |  | CELL UNITS DOOR MANUFACTURED HINGES TOP AND BOTTOM DRAWING POL 2012 /D4 TYPE A AS PER SET MINIMUM REQUIREMENTS purposed made adjustable units detail of cell door hinges: WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED Specifications and set minimum requirements: - Cell door, gate and frame adjustable hinges will be manufactured from Bright mild steel EN8 according to specifications as specified in Drawing POL 2012 / D4: DETAIL OF CELL DOOR HINGES. - Manufactured cell door, gate and frame adjustable hinges will be hot dipped galvanised in accordance with SANS Specifications 121 (SABS ISO1461). - Stainless steel bolt M12 x 30 (Setting bolt) and Stainless steel half locknut M12 stainless steel material. One Stainless steel setting bolt and one stainless steel locknut will be supplied and fitted accordingly to each hot dipped galvanised cell door, gate and frame hinge as a complete unit. - Stainless steel 6 mm grease nipple will be supplied and fitted accordingly to the hot dipped mild steel hinge after the manufactured hinge units were galvanised. | STATE: _____________ |
| 2.1.1 |  |  | STATE: _____________ |
| 2.2 |  | CELL UNITS’ DOOR MANUFACTURED HINGES MIDDLE DRAWING POL 2012 /D4 TYPE B AS PER SET MINIMUM REQUIREMENTS purposed made adjustable units detail of cell door hinges: WILL BE MANUFACTURED TO SET SPECIFICATIONS, DRAWINGS, SUPPLIED, DELIVERED AND OFFLOADED Specifications and set minimum requirements: - Cell door, gate and frame hinges will be manufactured from Bright mild steel EN8. - Cell door, gate and frame hinges will be manufactured from Bright mild steel EN8 according to specifications as specified in Drawing POL 2012 / D4: DETAIL OF CELL DOOR HINGES. - Manufactured cell door, gate and frame hinges will be hot dipped galvanised in accordance with SANS Specifications 121 (SABS ISO1461). - Stainless steel 6 mm grease nipple will be supplied and fitted accordingly to the hot dipped mild steel hinge after the manufactured hinge units were galvanised. | STATE: _____________ |
| 2.2.1 |  |  | STATE: _____________ |